# LATCH DOWN INSTRUCTIONS

Locate the two allen head screws in the 1 ½” slot midway on the top plate.  (See drawing below) (In some cases there will be only one allen head screw in the slot)  Loosen the screws and push both sections of teeth down with your hand taking care not to allow teeth to spring up and cause injury.  Hold the teeth down below the top plate and slide the allen head screw mechanism to the other end of the 1 ½” slot.  Tighten the screws securely.  The teeth should now be in a lock down position.  To unlock the teeth simply loosen the allen head screws taking care to keep hands and feet away from teeth.  The teeth will pop up very quickly.  Guardian Traffic Systems are not designed to be left in a latch down position for long periods.  The teeth and shafts are zinc plated but may still be susceptible to rust which can cause the units to freeze in a lock down position when left immobile for long periods.

Surface Mount #1230.100



## In Ground

#1130.100

#1160.100      #1167.100